BANGLADESH UNIVERSITY OF ENGINEERING AND TECHNOLOGY

Department of Electrical and Electronic Engineering

Course No 	: EEE 208                                                         	Group No :01

Course Title: Electronic Circuit II Laboratory 	              	Section: A1

# DIGITAL THERMOMETER USING OP-AMP AND BIPOLAR TRANSISTOR

Submitted to:

Dr. Md. Safiqul Islam

Professor

Safin-Bin-Hamid

Lecturer

Submitted By:

Name: Anudwaipaon Antu                                                 Name: Muzakkir Ahmed

Student ID: 1906001                                                          Student ID: 1906002

Level: 2; Term: 2                                                                Level: 2; Term: 2

Department: EEE                                                                Department: EEE

Date of Submission: 01-09-2022

Introduction:

The circuit of the project was done (digital thermometer) such that provides an output a voltage proportional to the temperature. It gives an output voltage of 10 mV per degree centigrade, when the temperature changes the voltage across the sensor (transistor in our case) will change since the transistor in the diode connection mode so VBE changes according to the changing temperature , when VBE changes, the output voltage will change linearly with the temperature Immediately.

In this project we kept IC constant and did temperature sweep. We get decreasing VBE with increasing temperature. We set the value of resistors to get in 00Celsius output voltage 0V and 1000 Celsius 10V.

Aim of the project:

- To earn a practical knowledge about temperature sensitivity of semiconductor junction

- To construct a circuit which gives us output voltage proportional to temperature

Simulation Software: Pspice 9.2

The components used:

- The Resistors

- Op-amp (uA741)

The Op Amps are used for different purposes. U3 is used for completing the loop with virtual ground in it. And the second Op Amp U4 is used for scaling purpose (The project will show 0 to 1 V with the temperature change of (0℃) to (100℃)).

- NPN Bipolar junction transistor

The transistor is used as a sensor. Which Vbe value will be change with respect to the change of temperature.

- Wires

- DC power supply

Proposed Circuit Diagram:

If we look closer to the circuit, we find that when the temperature changes, the base to emitter voltage will change in transistor and when V_be changes, the output voltage of the op amp will change depending on the value of temperature since V_be is changing with temperature.

We know collector current equation of a transistor is IC= IS(eVbenVT-1) where IS is the reverse saturation current. If the transistor is operating in the active region, then eVbenVT≫1. So, the equation becomes IC≈ IS eVbenVT . In our project we kept IC constant. Rearranging the equation, we can write VbeVT=n ln⁡(ICIS). Left side of the equation is constant as IC is constant. If we denote the constant terms of the equation as ‘k’, the equation becomes Vbe=kVT. So, Vbe is proportional to VT and it forms a linear relation. Here VT is our temperature dependent parameter and it decreases as temperature rises. So, Vbe also decreases as the temperature rises.

To get output voltage 0V at 0 degree Celsius and 1V at 100 degree Celsius, we calculate the resistance in following way.

.

Assuming Ib+Ic= 0.5mA (because we want to sweep Vbe with temperature)

Let Vin=5V, R1= VinIc= 50.5m=10k ohm

Using temperature sweep we get, Vbe(0℃) = 678.163 mV and

Vbe(100℃) =490.031 mV

Now,  Vp=0 at T= 00C

-Vbe×R3R3+R2  +  Vin×R2R3+R2=0

→ R2R3=VBEVin

Let, R3 = 50k ohm and VBE(0℃)=678.163 mV

So, R2=  VBE(0°)Vin× R3

=678.163m5×50k=6.7816k ohm

Therefore,

R1=10kΩ

R2=6.7816kΩ

R3=50kΩ

Then,

V0=(1+R4R5) Vp

Here, V0 = 1 at T = 1000C

1 = (1+R4R5) Vp

→R4R5 =1Vp-1

Let, R5 = 10k ohm and VBE(100℃)=490.031 mV

So, Vp(100℃)=-VBE(100℃)×R3R3+R2  +  Vin×R2R3+R2

=-490.031m×50k+5×6.7816k50k +6.7816k

=0.16566 V

So, R4=  (1Vp100℃-1)× R5

=10.16566-1×10k

=50.3646k ohm

So,

R4=50.3646kΩ

R5=10kΩ

Schematic Diagram on PSPice:

We performed a sweep from 00C to 1000 C,

Output:

From the graph, we can see that the output voltage is changing linearly with temperature. Here, we find, 197.713 mV at 200C, 396.051 mV at 400C, 596.143mV at 600C

Limitations:

- Every BJT, op-amp, MOSFET have a working temperature. Due to this reason, it has a limited range of measuring temperature.

- Due to the input bias voltage, offset voltage of op-amp, there may be some error to measure the temperature. If we used LM series op amp instead of UA741, we would get more precise result.

Future Work:

- We can use LM35 or LM35DZ which is a linear temperature sensor calibrated in kelvin.

- We can make a device which we can place where we want to measure temperature. Using volt-meter the device will measure voltage and thus measure temperature.

Applications:

- It can be used to measure the internal temperature of PC, cell phone, laptop etc.

- It can be used as a simple digital thermometer.

- To measure the temperature of digital devices, this project can be used.

Discussion:

In this project we developed a circuit that can measure temperature with respect to the change of voltage which works using the temperature sensing of transistor. Using this project, we can make device which can be used to measure the temperature of any device. We got a linear graph of temperature vs voltage plot. Using this graph, we can get digital output. Some development work can be done in future on this project.